Architecture Description

Warehouse Robots – Team 3

Software Engineering for Cyber-Physical Systems

2025–2026

Table of Contents

1. Introduction	3

2. Requirements	3

2.1  World–Machine Model	3

2.2  Functional Requirements	3

2.3  Stimulus → Response Specification	4

3. Evolution from Initial Design	4

4. System Overview	5

5. Robot Architecture	6

5.1  State Machine	6

5.2  Hardware Abstraction & Dependency Injection	7

5.3  Navigation	8

5.4  Odometry (Sensor Fusion)	8

5.5  Collision Avoidance	8

6. Server Architecture	9

6.1  Server Update Loop (100 ms)	9

6.2  Component Summary	9

7. Communication Architecture	10

7.1  Task Sequence	10

7.2  MQTT Topics  (QoS = 1, JSON)	10

7.3  REST API  (FastAPI)	11

7.4  WebSocket  (/ws)	11

8. Formal Specifications (TLA+)	12

8.1  Aisle Mutual Exclusion	12

8.2  Robot State Machine	12

8.3  Task Lifecycle	13

9. Key Design Choices	14

10. Conclusion	14

# 1. Introduction

This document describes the software architecture of the Warehouse Robots system (Team 3). The system coordinates a fleet of autonomous mobile robots that pick items from warehouse shelves and return them to a central base. Three architectural goals guided the design:

- Separation of concerns: robot control, server orchestration, and the web interface are independent layers

- Platform independence: robot logic is decoupled from Webots via abstract interfaces

- Resilience: lease-based locking, offline detection, and priority-based collision avoidance handle partial failures

# 2. Requirements

## 2.1  World–Machine Model

The environment and system phenomena are classified according to the World–Machine model:

| Category | Phenomena |
| --- | --- |
| World (physical env) | Warehouse aisles (one robot at a time), shelves, base location, mobile robots, WiFi network, human operators |
| Hidden env. phenomena (eh) | Mechanical wear, human presence in aisles, WiFi interference, hardware failures, floor obstacles |
| Visible env. phenomena (ev) | LiDAR distance readings, wheel encoder ticks, compass yaw, robot network connectivity |
| Visible system phenomena (sv) | Motor velocity commands, aisle lock grant/release messages, task assignment commands, heartbeat messages, task status updates |

## 2.2  Functional Requirements

| # | Requirement | Property |
| --- | --- | --- |
| 1 | Liveness | Every queued task is eventually picked and delivered |
| 2 | Collision freedom | Robots never collide with each other or obstacles |
| 3 | Aisle exclusivity | At most one robot is present in any aisle at any time |
| 4 | Return to base | Every robot returns to the base after each pick |
| 5 | Operator interface | Operator can manage the queue and monitor robots via web UI |
| 6 | Autonomous recovery | System recovers from connectivity loss without operator intervention |
| 7 | Portability | Deployable on real hardware with minimal code changes |

## 2.3  Stimulus → Response Specification

| Stimulus (ev) | Response (sv) |
| --- | --- |
| Distance reading < stop threshold | Immediate stop command issued to robot |
| Robot connectivity → offline | Motion stops; aisle lease expires server-side after 60 s |
| Robot requests aisle entry | Aisle marked locked; other robots denied access |
| Operator submits task via web UI | Navigation command issued to nearest available robot |
| Aisle lease expires (60 s without release) | Lock reclaimed; owning robot notified with granted=false |

# 3. Evolution from Initial Design

Key differences between the initial design (first presentation) and the current implementation:

| Initial design | Current implementation | Reason |
| --- | --- | --- |
| Shortest-path navigation algorithm | Proportional controller to fixed coordinates | Static layout makes path planning unnecessary |
| Local mapping as a system phenomenon | Static WarehouseMap with hardcoded coordinates | Fixed geometry; no runtime map needed |
| Microservices architecture | Single-process FastAPI with modular internal structure | Reduced operational complexity; modules still separate concerns |
| Strict hexagonal (ports & adapters) | Layered architecture with dependency inversion (DIP) | DIP fully applied; strict port naming not needed at this scale |
| Explicit safe state on connectivity loss | WAIT_CONNECTION only at startup; lease expires server-side | Mid-task loss handled implicitly via lease timeout |
| Sensor processing pipeline | Single front-sector LiDAR + compass | Complex fusion not needed in simulation |

# 4. System Overview

Figure 1 – Three-tier component architecture

| Tier | Directory | Responsibility | Protocol |
| --- | --- | --- | --- |
| Robot | src/ | State machine, navigation, sensor fusion, collision avoidance | MQTT |
| Server | server/ | Task dispatch, aisle management, robot registry, API | MQTT + HTTP/WS |
| Web Interface | webapp/ | Task submission, real-time fleet monitoring | REST + WebSocket |

# 5. Robot Architecture

## 5.1  State Machine

Figure 2 – Robot finite state machine

Each state implements IRobotState: on_enter(robot), on_exit(robot), update(robot, dt) → TransitionID (called every tick), and on_event(robot, event) → None (handles async MQTT events).

| State | Behaviour |
| --- | --- |
| WAIT_CONNECTION | Connects to MQTT broker; transitions to IDLE on success |
| IDLE | Stops robot; on TaskReceivedEvent stores task, requests aisle lock, publishes IN_PROGRESS |
| WAIT_FOR_AISLE_ACCESS | Halts; retries until AisleResponseEvent(granted=true); sets aisle_lock_granted flag |
| MOVE_TO_AISLE | Navigates to aisle entry point; transitions on arrival |
| MOVE_TO_SEGMENT | Navigates to shelf location; transitions on arrival |
| PICK_ITEM | Stops; waits 5–10 s (simulated pickup); transitions to MOVE_BACK_TO_AISLE |
| MOVE_BACK_TO_AISLE | Returns to entry; if aisle_lock_granted is false → stop in place (does not re-request) |
| MOVE_TO_BASE | Releases lock, navigates to base, publishes DONE, transitions to IDLE |

## 5.2  Hardware Abstraction & Dependency Injection

Figure 3 – Interface hierarchy and dependency injection via RobotFactory

The Robot class depends only on interfaces. RobotFactory creates a WebotsHardware instance (the low-level Webots device layer) and passes it to both WebotsMotionController and WebotsSensorsController. These controllers translate abstract interface calls into Webots device API calls. Collision avoidance is applied in each state's update() via robot.collision.apply(), not inside the motion controller.

ISensorsController exposes: get_scan(), get_front_distance(), get_left_distance(), get_right_distance(), get_rear_distance(), get_yaw(), get_gyro(), get_accelerometer(), get_wheel_positions(). Replacing WebotsHardware, WebotsMotionController, and WebotsSensorsController is sufficient for real-hardware deployment.

## 5.3  Navigation

Proportional controller (no PID); arrival threshold 0.02 m:

angle_error  = normalize(atan2(dy, dx) − pose.theta)

angular_vel  = clamp(k_angular × angle_error,  −max_ω, max_ω)

linear_vel   = min(k_linear × distance × cos(angle_error), max_v)

if |angle_error| > 0.8 rad:  linear_vel = 0   # rotate-in-place first

Parameters: k_linear = 1.0, k_angular = 2.5, max_linear = 0.20 m/s, max_angular = 1.5 rad/s.

## 5.4  Odometry (Sensor Fusion)

Wheel encoders provide position; compass overrides yaw to eliminate drift:

if |d_theta| < 1e-6:  x += d_center·cos θ;  y += d_center·sin θ   # straight-line

else:  x += r·(sin(θ+dθ) − sin θ);  y += r·(cos θ − cos(θ+dθ))  # circular arc

theta = compass_yaw  (if available)

## 5.5  Collision Avoidance

| State | Trigger | Effect |
| --- | --- | --- |
| SAFE | Path is clear | Full speed |
| EVADE | Obstacle detected and closing, or critically close | Reduced linear speed; left-turn bias added to navigator output |
| STOP | Obstacle critically close | Zero velocity; forced evasion window after recovery |

The CollisionManager uses front-sector distance readings combined with a closing-rate calculation to distinguish between stationary obstacles and approaching robots. A mandatory evasion window after a hard stop prevents the robot from immediately resuming full speed before the obstacle has cleared.

# 6. Server Architecture

## 6.1  Server Update Loop (100 ms)

- Process MQTT events → forward to RobotTracker, AisleManager, TaskStore; WebSocket bridge broadcasts immediately.

- RobotTracker.update() → marks robots OFFLINE if no heartbeat for 4 s.

- AisleManager.update() → expires leases > 60 s; notifies affected robot with granted=false.

- TaskDispatcher.try_dispatch() → assigns next pending task to nearest idle robot.

## 6.2  Component Summary

| Component | Responsibility |
| --- | --- |
| TaskDispatcher | FIFO task queue; nearest-idle-robot assignment (Euclidean distance to aisle entry) |
| AisleManager | Lease-based mutual exclusion (one robot per aisle); 60 s timeout for crash recovery |
| RobotTracker | Auto-registers robots on first heartbeat; OFFLINE after 4 s silence; OFFLINE state not cleared by heartbeat |
| TaskStore | Tracks full task lifecycle: PENDING → IN_PROGRESS → DONE / REJECTED |
| WarehouseMap | Static geometry: 5 aisles × 3 shelves; aisle entry X = 3.0 m; shelf X = 4.5 / 5.5 / 6.5 m |

# 7. Communication Architecture

## 7.1  Task Sequence

Figure 4 – End-to-end task sequence (Operator → Robot → Done)

## 7.2  MQTT Topics  (QoS = 1, JSON)

| Topic | Direction | Key payload fields |
| --- | --- | --- |
| robot/{id}/heartbeat | Robot → Server | robot_id, pose {x,y,θ}, task_id |
| robot/{id}/task/assign | Server → Robot | task_id, aisle_pos, segment_pos |
| robot/{id}/task/status | Robot → Server | task_id, status, reason |
| robot/{id}/aisle/response | Server → Robot | aisle_id, granted |
| aisle/{id}/request | Robot → Server | robot_id, task_id |
| aisle/{id}/release | Robot → Server | robot_id |

## 7.3  REST API  (FastAPI)

| Method + Path | Description | Response |
| --- | --- | --- |
| POST /api/tasks | Submit new task | 201 TaskSchema |
| GET /api/tasks | List tasks (filter by status) | 200 TaskSchema[] |
| DELETE /api/tasks/{id} | Cancel PENDING task | 204 / 409 |
| GET /api/robots | All robots + state | 200 RobotSchema[] |
| GET /api/aisles | All aisles + lock status | 200 AisleSchema[] |

## 7.4  WebSocket  (/ws)

Server-push events to the browser: robot_heartbeat, task_status, aisle_request, aisle_release. Eliminates polling; WebSocketEventBridge converts domain events to JSON and broadcasts to all connected clients.

# 8. Formal Specifications (TLA+)

Three TLA+ modules capture the key safety and liveness properties.

## 8.1  Aisle Mutual Exclusion

---- MODULE AisleMutualExclusion ----

EXTENDS Integers, FiniteSets

CONSTANTS Robots, Aisles

VARIABLES aisle_owner, robot_state

NULL == CHOOSE n : n \notin Robots

TypeOK ==

/\ aisle_owner \in [Aisles -> Robots \cup {NULL}]

/\ robot_state \in [Robots -> {"IDLE","WAIT_AISLE","IN_AISLE","RETURNING"}]

(* Safety: at most one robot holds any aisle lock *)

MutualExclusion ==

\A a \in Aisles : Cardinality({r \in Robots : aisle_owner[a] = r}) <= 1

RequestAisle(r, a) ==

/\ robot_state[r] = "WAIT_AISLE" /\ aisle_owner[a] = NULL

/\ aisle_owner' = [aisle_owner EXCEPT ![a] = r]

/\ robot_state' = [robot_state EXCEPT ![r] = "IN_AISLE"]

ReleaseAisle(r, a) ==

/\ aisle_owner[a] = r

/\ aisle_owner' = [aisle_owner EXCEPT ![a] = NULL]

/\ robot_state' = [robot_state EXCEPT ![r] = "RETURNING"]

ExpireLease(a) ==  (* server reclaims lock after 60 s timeout *)

/\ aisle_owner[a] # NULL

/\ aisle_owner' = [aisle_owner EXCEPT ![a] = NULL]

/\ UNCHANGED robot_state

====

## 8.2  Robot State Machine

---- MODULE RobotStateMachine ----

VARIABLES state, has_task, aisle_granted

States == {"WAIT_CONNECTION","IDLE","WAIT_FOR_AISLE_ACCESS",

"MOVE_TO_AISLE","MOVE_TO_SEGMENT","PICK_ITEM",

"MOVE_BACK_TO_AISLE","MOVE_TO_BASE"}

Init == /\ state = "WAIT_CONNECTION" /\ has_task = FALSE /\ aisle_granted = FALSE

(* Safety: robot never enters aisle without holding the lock *)

NeverEnterWithoutLock ==

state \in {"MOVE_TO_AISLE","MOVE_TO_SEGMENT",

"PICK_ITEM","MOVE_BACK_TO_AISLE"}

=> aisle_granted = TRUE

(* Liveness: every assigned task eventually completes *)

TaskEventuallyDone == has_task ~> (state = "IDLE" /\ ~has_task)

====

## 8.3  Task Lifecycle

---- MODULE TaskLifecycle ----

EXTENDS FiniteSets

CONSTANTS Tasks, Robots

NULL == CHOOSE n : n \notin Robots

VARIABLES task_status, task_robot

Init ==

/\ task_status = [t \in Tasks |-> "PENDING"]

/\ task_robot  = [t \in Tasks |-> NULL]

Dispatch(t, r) ==

/\ task_status[t] = "PENDING" /\ task_robot[t] = NULL

/\ task_status' = [task_status EXCEPT ![t] = "IN_PROGRESS"]

/\ task_robot'  = [task_robot  EXCEPT ![t] = r]

(* Safety: a task is assigned to at most one robot *)

SingleAssignment ==

\A t \in Tasks : task_status[t] = "IN_PROGRESS" => task_robot[t] # NULL

(* Safety: DONE and REJECTED are terminal states *)

NoRollback ==

/\ [][task_status[t] = "DONE"     => task_status'[t] = "DONE"]_task_status

/\ [][task_status[t] = "REJECTED" => task_status'[t] = "REJECTED"]_task_status

====

# 9. Key Design Choices

| Design Choice | Alternative | Rationale |
| --- | --- | --- |
| State machine for robot logic | Monolithic loop | Single responsibility per state; extensible without modifying existing states |
| Abstract interfaces + factory (DIP) | Direct Webots dependency | Robot core has zero Webots imports; swap two classes for real hardware |
| MQTT pub-sub (QoS = 1) | Direct TCP / HTTP polling | Scales to N robots; handles WiFi dead spots; wildcard subscriptions |
| Lease-based aisle locking (60 s) | Full path planning | Simple crash recovery; no extra protocol needed |
| Priority-based collision avoidance | MQTT negotiation protocol | Deterministic, no extra messages, works without server |
| Proportional controller (no PID) | PID controller | Sufficient for straight aisles; fewer tuning parameters |
| Thread-safe EventQueue | Direct callback into FSM | State machine stays single-threaded; only shared structure with MQTT thread |
| Nearest-idle-robot dispatch | Optimal scheduling | O(n) simplicity; adequate for proof-of-concept; trivially replaceable |
| Sensor fusion (compass + encoders) | Encoders only | Compass corrects cumulative yaw drift |
| Vanilla JS SPA (no framework) | React / Angular | No build step; sufficient for monitoring UI scope |

# 10. Conclusion

The Warehouse Robots architecture achieves clean separation across three independent tiers. The robot tier is fully platform-independent — its only Webots coupling is two concrete interface implementations. The server tier coordinates the fleet through lightweight, formally verified mechanisms. The design deliberately favours simplicity over premature optimisation; each algorithmic choice is behind an interface or a single method, making it straightforward to substitute more sophisticated solutions.